## ELECTRICAL SPECIFICATION

Unit Supply Voltage: 8V - 16V

Unit Pump Supply Voltage: 40V-48V

12V Supply Current: <200mA (92uF input capacitance)

48V Supply Current 10A Max, (Current limited with 200uF input capacitance)

Supply reverse and short circuit voltage protection

Peak Current (Pull-in Current + Hysteresis): Up to 7.5A

Peak Hold (Hold Current + Hysteresis): Up to 2.8A

Analogue output for pump current diagnostics:

- 0-5V, 3.062A/V

Pump driver output in bridge configuration

- Assumes a 0.5R, 2.2mH load.

- Designed for Bosch HDP 6 fuel pump

Diagnostics provided via CAN:

- Board Temperature

- Peak Current

- Unit Supply Voltages

- Trigger Count

- Minimum Pulse Width Violation flag

- Under-voltage Warning flag (<38.6V)

- Pump Open Circuit flag

## PRODUCT DESCRIPTION

The HPF-320 is a compact high-pressure fuel pump driver unit for motorsport applications, designed to operate alongside a powertrain control unit. The unit generates a complex current controlled waveform required to activate the pump's solenoid inlet valve directly from a single TTL trigger pulse. The ON trigger pulse from the powertrain control unit commands the total pulse width, whilst the pull-in time, pull-in current, holding current, pull-in hysteresis, and holding hysteresis are defined over CAN.

A single-chip processor is used to set up the waveform and to gather diagnostic information which is transmitted on CAN to the powertrain control unit. An FPGA accepts the external trigger pulses and controls the drive stage based on the waveform configuration from the processor.

## ENVIRONMENTAL

Vibration 24 hours in 3 axes

| Frequency (Hz) | Amplitude (g2/Hz) |
| --- | --- |
| 100 | 0.00797 |
| 200 | 0.171757 |
| 300 | 0.54279 |
| 650 | 0.00965 |
| 1000 | 0.0208 |

Shock test: 50g, 1½ sine, 11ms duration, 5 times per axis.

Sealed and pressure tested to ±30mBar

Operating temperature -10 to 85°C.

Resistant to standard motorsport fluids.

Designed and manufactured to incorporate the 'essential protection requirements' of EMC Directive 2004/108/EC.

## DRIVE OUTPUT CHARACTERISTCS

Pump start and stop controlled via 1 x external trigger input.

Waveform configuration controlled with CAN. After initial configuration, the latest waveform will be recalled after each power cycle.

15,000 rpm maximum engine speed

Minimum trigger width 127µs

Closed-loop current control using PWM

If open circuit or overcurrent flag is set the output waveform will be terminated and the unit will cease firing until these flags are resolved.

The unit provides a current-controlled waveform as follows:

## CONNECTION DEFINITION

Connector 8STA2-1035PN

Mating Part number: 8STA6-1035SN

| Pin | Signal |
| --- | --- |
| 1 | CAN High |
| 2 | CAN Low |
| 3 | Input - ON trigger |
| 4 | Ext Digital Ground Ref |
| 5 | Pump Current Output |
| 6 | 12V Ground |
| 7 | 12V Supply |
| 8 | 48V Ground |
| 9 | 48V Ground |
| 10 | 48V Supply |
| 11 | 48V Supply |
| 12 | Output - HPFP Return |
| 13 | Output - HPFP Send |

## MECHANICAL

See attached drawing D100 520 003 018

CAD Model D016 999 002 580

Weight <115g.

Aluminium alloy body, hard anodised and dyed black.